High Speed 4-Port Web Server Products Series

User Manual

&

Installation Guide

Verson:1.0

Table of Contents

TABLE OF CONTENTS…………………………………………..……………………………………2

1. PRODUCT OVERVIEW…………………………………………………………………………..3

What Is 4-Port Web Server…………………………………………………………..………..3

Product Features…………………………………………………………….…….……………………3

Application Architecture…………………………………………………….………………………3

2.PHYSICAL ONNECTIONS…………….…………………………….………………………….5

Product Perspective Graph……………………………………………………….……………….5

3. INSTALLATION………………………………………………………….…………………………. 6

Before You Start……………………………………………………….….………………………….. 6

Network Connectivity Check……………………………………………………...…………….7

Connect Your CCTV Camera………………………...…………………………….……….……8

Connect Web Server To Network……………………………..…………………….………8

Connect Power Supply………………………………………………….…….…………………….9

Start Your First Time In Networking………………………..………………………….….10

How To Visit The Built-in Web……………………………………..……………….………….11

About The User Interface of Web Server….……………………………………………12

How To Check The Firmware Version…………………………………..……………….…13

How To Change The Configurations.……………………………………………….…….…14

User Management………………………………………………….……….…………………………17

Log……………………………………….………………………………………………….…………………18

Network……………………………………….……………………………….…………………………..19

Video (By Channel)………………………………………………..……….……………………….21

PTZ Camera…………………………….……………………………..…………………………….….22

Event……………………………………….………………………………….………………………….…23

4. PRODUCT SPECIFICATIONS……………………………….……………………………..28

5. APPENDIX………………………….…………………………………………………………….……29

Dictation………………………….……………………………………….…………………….…………29

1. Product Overview

##### What Is 4-Port Video Server

4-Port Web Server is designed for intranet & internet. Users can view image or change configurations via built-in web server. The two-layer user-validation prevents any illegal user to monitor or change configuration once user validation is enabled

##### Product Features

| High Speed 4-Port Web Server | - CPU: 32 Bits RISC Processor. 16Mb Flash, 128Mb SDRAM - Linux OS - TCP/IP network remote Video transmission system - High performance JPEG/MJPEG compression - Built-in Web server - Built-in DHCP Client - Built-in Motion Detection - 3external composite video input (DI x 2, DO x 1) - Frame Rate Control - Administrator / User level password protection - Snapshots - On-line firmware upgrade - Frame Rate: 1 channel input: NTSC: 30 frame/sec (320x240) PAL: 25 frame/sec (352x288) 4 channel input: 22 frame/sec/channel (NTSC: 320x240/PAL: 352x288) |
| --- | --- |

##### Application Architecture

High Performance 4-Port Web Server

The following picture represents standard application architecture for 4-Port Web Server.

# 2. Physical Connections

## Product Perspective Graph

Front Panel Connections

VINx: Video Input (1 ~4)

Pin Assignment:

Ethernet: UTP Cable

Power: DV 5V

# 3. Installation

Before You Start

Before you proceed to the installation steps of the 4-Port Web Server. Please check the content of the package. Which must include all the components lists below.

1. 4-Port Web Server

2. Power Adapter

3. User manual & installation guide

4. Quick installation reference card

Please check the hardware/software requirements of the Web Server monitoring workstation. You may download or update your system to the newer version to make the monitoring components work.

##### Network Connectivity Check

Please check your network connectivity before you start to install the product. Confirm the link status of your LAN is OK. You may try to use the following way to check the network.

Assuming you are under Microsoft Windows 2000 desktop, Click on Start button, Programs, Accessories, Command Prompt，Then there will be a blank window appear to wait command input. Please key in the following command to test the network status.

C:\>ping 192.168.1.1 [Enter]

*Note: the target IP address given above may be correct in common scenarios, if your network is not configured by IP segment 192.168.1.x, please change the IP Address to a valid address in your local network. Any difficulties please contact your network administrator for assistance.

If your network status is OK, the result of above command should like following

Pinging 192.168.1.1 with 32 bytes of data:

Reply from 192.168.1.1 bytes=32 time=10ms TTL=64

…

If you have confirmed your network connectivity is OK, Please proceed to next part of installation guide, otherwise, contact your network administrator to recover the problem.

##### Connect Your CCTV Camera

Locate the Video Input connector (Marked in VinX) at the front panel of the Web Server and plug all the CCTV cameras to the connector by using a video signal transmission cable; finally, lock the BNC connector correctly.

##### Connect Video Server To Network

Connect Web Server to your hub/switch by using a normal RJ45 cable, plug the RJ45 cable into the Ethernet connector locate at the rear panel of the Web Server, another side connect to your hub/switch.

##### Connect Power Supply

Connect the Web Server and power source with the adapter provided, plug the power adapter into the DC5V Connector. Once the power status is OK, the LED in red located at the rear panel of Web Server should light up.

##### Start Your First Time in Network Monitoring

Run the “IPLocator30” utility, the newly connected Web Server will be listed in the application window. Please remember its’ IP address.

Open an Internet Explorer Window input the following address into the address textbox:

http://[Web Server IP Address provided by IPLocator30]

After you press on the enter key, the main working interface and video picked by the camera should appear after a short period. The installation steps are completed. You may now start your first network monitoring experience. You may change the video channel by click on channel switch located at the left bottom of the screen.

Note: when you first time enter the working interface, the browser may ask whether to install a COMWebSurv component, which is used to communicate with our Web Server product, PLEASE CHOOSE YES to install, otherwise the monitoring function may not work.

Launch your Internet Explorer (I.E. 6.0 or above) first, and then type the Web Server’s url. (i.e. http://192.168.1.100). The user validation is disabled as default, no login is necessary. The user is same as the system administrator (Default : root/pass).

##### How To Visit The Built-in Web

For the first time user visits the Web Server, the ActiveX (ATLWebSurvCOM) will be downloaded to the [Downloaded Program File]. Be sure to click the button [Yes] as below.

Notes: Call your system administrator if you have not enough privilege to download the ActiveX. (i.e. Users should be the administrator to download the ActiveX for Windows 2000/XP operation system)

You can find the ActiveX ATLWebSurvCOM will be downloaded to the [Downloaded Program File] folder. The above dialog will pop automatically once new ActiveX is available(i.e. after firmware upgrade)

The upper window is for company logo display (i.e. This is a generic version. The company logo leaves blank). The left side of the window is the control panel. It is used for PTZ control, Dome preset point recall and image resolution change. The right window is for image display. (Please switch to full screen mode when the resolution is set to VGA mode)

##### About The User Interface of Video Server

Click the Web Server icon and you will read the firmware version

##### How to Check The Firmware Version

The system administrator: root is the one who has the privilege to change system configuration if user validation is enabled. The following window will appear when you click [Options] icon in the control panel (left side of the window). The follow is the description & field definition.

##### How to Change The Configurations

[System]:

Server Info: Server Model and Firmware Version

Device Name: the name of hardware device.

Video Mode: Video Mode (NTSC or PAL)

Time Zone: Time Zone Setting

Display Option

Show Camera Name: Show Camera Name on Display

Show Date Time: Show Date Time on display

Traffic Control: Bandwidth Setting

Others

Load default configuration: Load the factory default.

Reboot: Reboot the hardware remotely.

Firmware Upgrade : Upload & upgrade the Web Server firmware.

[Firmware Upgrade]: Please follow the standard procedure to upgrade firmware.

Step 0: Stop all unnecessary running application in your PC.

Step 1: Uncheck the [Browse..] and [Upload] motion images in the Motion Detection Setting and reboot the hardware. Note: You have to make all event setting to be disable, before you do this step.

Step 2: Set Image Quality to normal & Resolution to 352x288 or 320x240(or below)

Step 3: Reboot your Device

Step 4: Upload the firmware: Be sure to upload the correct firmware. Wrong firmware may cause your Web Server malfunction. It may take a few minutes to upload the firmware to memory depending on the network environment. Since too many unknown situation may occur in the Internet, we strongly recommend you to upgrade firmware in Intranet.

Step 5: Write firmware into Flash. It takes about 2 minutes to write firmware for the whole process. Please click [OK] button when the complete dialog pops up and the Web Server will reboot automatically. DO NOT reboot the device manually.

Note: Any interrupt during writing process will cause the Web Server malfunction.

The administrator (root) has the privilege to manage users if User Validation is enabled. Here are the configurations available & field definition.

##### User Management

[User]:

User validation

Yes: Enable user check

No: Disable user check

Add/Modify User: To create new user or modify existing user password

Username: user name

Password: user password

Confirm: user password confirmation

Delete user: To delete existing user

Current User List: The users in the Web Server

The system log will be listed in the window when you click [Log] icon.

##### Log

Operation: The operation which user operates

Value: The value to change

Client IP: The IP address of client PC

Client MAC: The mac address of client PC

Date time: date-time which operation occurs

Allows administrator (root) to change network setting when you click [Network] icon.

##### Network

IP Setting: Please consult you networking manager for the following values

Manual: To set network setting manually.

DHCP: To let Web Server get network setting from DHCP server automatically.

IP Address: IP address of Web Server

Subnet mask: Subnet mask of Web Server

Default Gateway: Default gateway of Web Server

DNS 1/2/3: DNS Server IP if necessary

Http Port: Change the http port of built-in web server (default 80).

(i.e. range 80, 1025 ~ 65535)

COM Port Setting:

COM Port: COM1 only

COM Port Mode: RS232/RS485

Baud Rate: 300 ~115200

Parity: None/Odd/Even

Data Bits: 5/6/7/8

Stop Bits: 1/2

Allows administrator (root) to change Image Setting, Traffic Control or Motion Detection.

Video (by Channel)

Image Setting

Quality: Image Quality (Low/Normal/Good/Fine/Excellent)

Images Size: 160x120/176x144/320x240/352x288/640x480/704x576

Rotation: Nomal/180 degree (Upside down)

Advanced: Image preference adjustment

Brightness: Adjust image brightness

Contrast: Adjust image contrast

Hue: Adjust image hue

Saturation: Adjust image saturation

Traffic Control

Unlimited: To send out image at full speed. (up to 30 frames per second)

Frame per second: Delay to send image every x second (mini-second)

Allows administrator (root) to configure PTZ Dome. Please select the right model for your dome to make it work properly.

PTZ Camera

Dome Setting:

Dome Model: Dome model list which Web Server supports

Dome ID: ID for RS-485

Preset: The preset position of Dome camera

Location: The preset location name of Dome camera

Speed: Dome speed for Pan/Tilt/Zoom operation

This Option contains three major functions: Server Configuration, GPIO Configuration and Event Configuration

###### Event

Server Configuration: User can setup Ftp and Mail services that provides the actions in

See figure below:

Allows administrator (root) to configure the mail & ftp setting once motion detected

Mail Setting: if check on Mail motion images, Device will send motion images

Mail Server: Name or IP of Mail server

Username: Username

Password: User password (Password will be validated if checked)

Mail From: Mail sender

Mail To: Mail receiver

Mail Subject: Mail subject

FTP Setting: if check on Mail motion images, Web DVR will upload motion

images

FTP Server: FTP Server

Username: Username

Password: User password

Upload Path: Images upload path

GPIO Configuration: Allow user to define types of digital input and their status.

See Figure below:

DI Configuration:

Index: the Index of digital input (range from 0-1)

DI Type: define type of digital input to : [Normal Open] or [Normal Close]

DI Status: Disable the DI or Enable it.

DO Configuration:

Index: the Index of digital output

DO Type: define type of digital input to : [Normal Open] or [Normal Close]

DO Status: Disable the DO or Enable it.

MSG Setting:

Alert Message function provides user the ability to send message to a remote server. Device manager can assign ip address, port number and message for remote server. Thus, Remote server can collect related alert messages via network when events are triggered.

[IP Address]: Server’s IP address.

[Port Num]: Http port Number.

[Message]: Message string.

Event Configuration: User can select Event Index to locate Event (range from 0-2) and also decide to enable or disable this event. Three parts of event setting will be discussed below: [Trigger condition], [Delay time], and [Trigger Action]

[Source]: The [Input Type] contains DI0, DI1 and Motion Detection

[Enable]: Select to Enable the Input Type that you selected.

[Delay time]: Duration that ignores same event trigger. Range from (1-30) sec

[Action]: Including FTP, MAIL, DO and MSG.

After System accepts trigger condition, related actions can be launched. FTP and MAIL will send image to remote area, thus; [Camera] is necessary for user to specify. DO is related to its index to specify which digital output (Only 0 is enable). MSG can send a string to remote server via TCP protocol which will be released soon.

# 4. Product Specifications

| Items |  | Specifications |
| --- | --- | --- |
| Description |  | High Speed 4-Port Web Server |
| Power | Supply | 5VDC, 2A |
|  | Consumption | 10 Watts |
| Networking | Connectivity | 10/100 Ethernet |
|  | Protocols | SMTP, FTP, DHCP, HTTP |
|  | Internet | Built-In HTTP Web Server, DHCP Client |
| Event Controls | Trigger By | Motion Detect, DI |
|  | Alarm Notifications | FTP/Mail motion image, DO, Message |
| Video Input | Signal | Composite 1.0 Vpp at 75 BNC |
|  | Standard | NTSC/PAL Auto Detection |
| Video Output |  | N/A |
| Images | Compression | JPEG/MJPEG |
|  | Flow Control | Frame-Rate Controls |
|  | Format | NTSC: 640x480/320x240/160x120 PAL: 704x576/352x288/176x144 |
|  | Frame Rate | Single Channel 30 fps @ 320x240 25 fps @ 352x288 |
|  |  | 4-Channel: 22 fps @ 320x240 20 fps @ 352x288 |
|  | Image adjustments | Brightness / Saturation / Contrast / Hue |
| Security Controls | Access Control | Administrator / User Level Password Protection |
|  | Configuration | Http Port configurable |
|  | Log | Historical Access Log Management |
| Serial Controls |  | RS-485 |
| GPIO | Input | 2(opto-isolated) |
|  | Output | 1(Relay) |
| Firmware Upgrade |  | On-Line Firmware Upgrade |
| Operating Temperature |  | 5oC~50oC(45oF~125oF) |
| LED Indications |  | Power, Network, Link, Network Transmission |
| Dimension |  | 202mm(W) x 35mm(H) x 110mm(L) |
| Remote Viewing |  | Windows 98/2000/XP IE6.0 or above |

# 5. Appendix

Dictation

TCP/IP:

Transport Control Protocol/Internet Protocol, the protocol used to transmit data between hosts in Internet, which provides data verification and error protection

Web:

World Wide Web service is used on Internet widely, It’s provides interactive rich content by using a web browser

Firmware:

A program built in hardware, which is used to control the hardware, provides software level functions

MJPEG:

Motion JPEG compression algorithm, use to compress digital video

MPEG:

International standard video compression algorithm

DHCP:

Dynamic host configuration protocol, used on IP network perform IP address, gateway, DNS server assignment, etc for client computers

DNS:

Domain Name Server/Service is used to translate domain name in to IP address

LAN:

Local area network

WAN:

Wide area network, normally Internet

FTP:

File transportation protocol, used to transmit files between server and client

POP:

Post office Protocol provides Email storage and receive service

SMTP:

Simple mail transportation protocol, provides Email delivery and relay service

Hub:

Physical layer network inter-link device, which simply connect ports together and broadcasting the data packet to all the port.

Switch:

Data link layer network inter-link device, which provides end-to-end data packet forwarding,

Router:

Network layer device, which connect networks and provides the function named “Routing” to find the best way to transport data from source to destination

CGI:

Common gateway interface, a kind of program, running on a web server to provides dynamic data process function and interacts with users’ action

ActiveX:

Concept and standard gave by Microsoft, which separates program into functioning pieces named ActiveX Control. These control can be embed into users’ program to give their specific function.