PRODUCT SPECIFICATION

VISUS CITY Series Enclosure

## Key Features

PATENTED H1B1 ENCLOSURE AND MOUNT

Attractive styling Design

- High Reliability Die-Cast

Aluminum Alloy Construction

- Cable Entry with IP68 Glands

- High Wear / UV Resistant

###### Rubber Seal

- Stainless Allen-head Screws for

Tampering and Rust prevention

- 3 mm Temper glass window with

High Reliability Gel

- Weatherproof to highest level IP68

- Robust Cable management bracket assembly

- Designed for Indoor/Outdoor Applications

- Easy Access for Servicing and Installation

- Fused input terminal block

- Models Available with Factory-Installed

Heater and Blower with Multiple voltage option

The VISUS CITY Series comprises a stylish pressure die cast enclosure, sized to suit CCD cameras with fixed focus lenses, plus wall bracket with cable management system.

The equipment has a compact body length of 13.38 (base) and 14.76 (overall) inches (34.0 and 37.5 cm, respectively).

VISUS CITY Series is designed to provide the installer with a secure and reliable camera option.

The internal camera cavity is simply accessed by loosening 3 stainless Allen screws. Two adjustable glands (PG13.5) on the bottom of the enclosure allow for easy installation of the power and video cables.

VISUS CITY | 1/3

TECHNICAL DATA

H1B1 Series Enclosure

TECHNICAL DATA

VISUS CITY Series Enclosure

| GENERAL |  |  |
| --- | --- | --- |
| Construction | Entirely made of die-cast aluminum alloy |  |
| Finish | RAL 9010 color of UV rays resistant powder coat |  |
| Environment | Totally protected against dust and any water from all directions |  |
| Weight |  |  |
|  | VISUS CITY 124H 5.25 lb (2.38 kg) |  |
|  | VISUS CITY 230H 5.25 lb (2.38 kg) |  |
|  | VISUS CITY 12HB 5.68 lb (2.58 kg) |  |
|  | VISUS CITY 24HB 5.68 lb (2.58 kg) |  |

| MECHANICAL |  |
| --- | --- |
| Window 4 mm thick tempered glass 82 x 68 mm ( W X H ) in viewing |  |
| Enclosure Length base 340 mm overall 375 mm Width 145 mm, Height 110 mm |  |
| Internal usable area 250 mm length 110 mm Width 70 mm Height |  |
| Certification Meets international IP68 standard |  |
| Camera mount Slot plate for adjustable camera positioning |  |
| Certification Meets international IP68 standard |  |
| Cable Entry 2 X PG 13.5 IP68 rating cable glands |  |

VISUS CITY | 2/3

TECHNICAL DATA

VISUS CITY Series Enclosure

| MECHANICAL |  |
| --- | --- |
| Bracket Robust wall mounting design , Length 330 mm Fully adjustable swivel head by 2 stainless steel allen-head screws Very good resistance to swaying Tilt angle of +40° to -150°; Pan angle of +100° to -100° Load rating: 76 lb (30kg) |  |
| ELECTRICAL |  |
| Heater control Below 15°±3°C automatic turning on Above 25°±3°C automatic turning off Blower control Above 35°±3°C automatic turning on Below 25°±3°C automatic turning off |  |
| Terminal block Fuse input ( Max. 400V ) |  |

VISUS CITY | 3/3

| VISUS CITY Series |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MODEL | Basic Enclosure & Mount | Heater 12Vdc / 24Vac | Heater 100 ~ 240Vac | Blower 12Vdc | Blower 24Vac | Max. Consumption |
| VISUS CITY 124H | ◆ | ◆ |  |  |  | 30W |
| VISUS CITY 230H | ◆ |  | ◆ |  |  | 40W |
| VISUS CITY 12HB | ◆ | ◆ |  | ◆ |  | 34W |
| VISUS CITY 24HB | ◆ | ◆ |  |  | ◆ | 34W |