# YOKO SPACE-DOME IR-LED Camera

#### MODEL: RYK-2G49LVF3/1&RYK-2G85LVF/1                                       2007/8/14

Top View

Side View

87.4mm

#### FEATURES

- Color camera with SONY 1/3“ Super HAD CCD sensor.

- Employs Digital Signal Processor (DSP) IC for image control.

- Pixel number: Normal Resolution : NTSC=270K / PAL=320K pixels

High Resolution   : NTSC=380K / PAL=440K pixels.

   High sensitivity, low smear, high anti-blooming.

- Auto Electronic Shutter (AES), Auto Gain Control (AGC), Auto Iris (AI), Back Light Compensation (BLC), Auto White Balance (AWB), Flickerless mode (FL).

- Built-in Vari-focal type DC-drive auto iris lens.

   Furnished with power cable and video cable.

- Only DC Voltage, low power consumption.

- Easy to install, easy to use, aesthetic.

#### SPECIFICATIONS ( Note: Design and specifications are subject to change without prior notice. )

| Model No. | RYK-2G49LVF3/1 | RYK-2G85LVF3/1 |  |
| --- | --- | --- | --- |
| Image Device | SONY 1/3“ Super HAD CCD sensor. | SONY 1/3” Hi-Res. HAD CCD sensor |  |
| Picture Elements | NTSC: 510492, PAL: 500582 | NTSC:768*494 , PAL:752*582 (H*V) |  |
| Horizontal Resolution | 380 TVL | 470 TVL |  |
| Minimum Illumination | 0.2 Lux / F2.0(Day), 0 LUX (IR ON) | 0.3 Lux / F2.0(Day), 0 LUX (IR ON) |  |
| S/N Ratio | More than 48dB |  |  |
| Auto Electronic Shutter | NTSC: 1/60s~1/100,000s, PAL: 1/50s~1/110,000s ON/OFF switchable. |  |  |
| Flickerless Mode | NTSC: 1/100s, PAL: 1/120s ON/OFF switchable. |  |  |
| Gamma Characteristic | 0.45 |  |  |
| Iris Control | DC Drive |  |  |
| Lens Furnished | Vari-Focal with DC Drive Iris Lens. VR3：Vari-Focal Lens with ICR f3.3~12.0mm/F1.6~360,DC Drive |  |  |
| Focal Length & Aperture Ratio | f = 3.7~12mm / F1.6~3.0 |  |  |
| Angle Field of View | Wide:71.6°×55.8°, Tele:23.8°×17.9° (H×V) |  |  |
| White Balance | Auto |  |  |
| Gain Control | Hi /Low |  |  |
| Back Light Compensation | ON/OFF |  |  |
| Synchronous System | Internal, Negative sync. |  |  |
| Video Output | 1 Vp-p / 75 Ohms. |  |  |
| Infrared Luminary | 24 pieces IR-LED,Life:600hrs |  |  |
| Wavelength | 850nm |  |  |
| Illuminate Distance | 20M |  |  |
| Power Supply | DC12V ±10％ |  |  |
| Power Consumption | IR On : 425mA(max) IR Off : 150mA(max) |  | IR On : 425mA(max) IR Off : 200mA(max) |
| Operating Temp. | -10℃ to 50℃ (14℉ to 122℉) |  |  |