# YOKO 1/3” COLOR CCD with IR-LED CAMERA

#### MODEL: YK-2V49L5            		    	                             2006/5/11

FRONT VIDW                        SIDE VIEW                           REAR VIEW

#### FEATURES

- Base on the SONY 1/3“ Super HAD CCD sensor Color camera.

- Employs Digital Signal Processor (DSP) IC for image control.

- Total pixels of sensor: NTSC=270K / PAL=320K pixels.

- High sensitivity, low smear, excellent anti-blooming, and high S/N ratio.

- Supports functions: Auto Electronic Shutter (AES), Auto Gain Control (AGC), Auto White Balance (AWB).

- Economical solution, Includes Board Lens and furnished with mounting bracket.

- AC100～240V, AC24V & DC12V type power request for optional.

- Built-in an infrared illuminant to assist for cameras in low illumination environment.

- Plug & Play. Installs easily and is suitable DIY (Do It by Yourself).

- Long life luminary with using special wavelength Infrared LED.

- Single power and signal connecting wire.

- Built-in environment illumination detection and control circuit that can turn-on/off the infrared illuminant automatically.

- Waterproofing criterion: IP66

#### SPECIFICATIONS (Note: Design and specifications are subject to change without prior notice.)

| Image Pick-up Device | 1/3“ Color SONY Super HAD |
| --- | --- |
| Effective Picture Elements | NTSC: 510492, PAL: 500582 (HV) |
| Horizontal Resolution | 380 TVL |
| Minimum Illumination | 0.2 LUX / F2.0 |
| S/N Ratio | More than 48 dB |
| Auto Electronic Shutter | NTSC: 1/60s~1/100,000s, PAL:1/50s~1/110,000s. |
| Auto White Balance | Color Temperature: 2500K~9500K. |
| Scanning System | Interlace 2:1 |
| Gamma Characteristic | 0.45 |
| Lens Furnished | Board Lens 12 mm / F2.0 (optional: 16 mm) |
| Synchronous System | Internal, Negative sync. |
| ##### Video Output | 1 Vp-p / 75 Ohms. |
| ##### Power consumption | 1.5 Watt Max.CCD Camera only. |
| Infrared Luminary | 84 pieces IR LED, Life: 6000 Hrs. |
| Wavelength | 850 nm. |
| Beam Spread Angle | 18° |
| Illuminate Distance | 50 M |
| Illuminator Active Limit | 10 Lux. |
| Power consumption | IR On : 17.5W max ; IR Off : 1.5W max |
| Power Supply | DC 12V±10% , AC24V , AC 110-240V (option) |
| Operating Temp. | -10℃ to 50℃ (14℉ to 122℉) |
| Waterproofing criterion | IP 66 |