# RYK-2698 Smoke Detector

Features：

．700 TV Line

．Support AWB, AGC, AES and DNR

．Built in Cone Lens

Dimension：

Specifications:

| Model No. | RYK-2698 |
| --- | --- |
| Image Device | 1/3" Color Sony Super HAD II CCD |
| Picture Elements(HxV) | NTSC: 976x494, PAL: 976x582 |
| Resolution | 700 TVL |
| Min. Illumination | 0.02 Lux @ F1.2 |
| S/N Ratio | More than 50dB |
| Electronic Shutter | NTSC:1/60~1/100,000, PAL:1/50~1/100,000 |
| Gamma | 0.45 |
| Iris Control | Fixed |
| Lens Furnished | Cone Lens 3.7mm / F2.0 |
| Sync. System | Internal |
| Video Output | 1 Vp-p / 75 Ohms |
| Power Supply | DC12V±10% |
| Power Consumption | 95mA |
| Operating Temp. | -10°C ~ 50°C |

# RYK-2598S PIR Detection

Features：

．700 TV Line

．Support AWB, AGC and AES

．Built-in Cone Lens

．Optional: PIR Module

Dimension：

Specifications:

| Model No. | RYK-2598S |
| --- | --- |
| Image Device | 1/3" Color Sony Super HAD II CCD |
| Picture Elements(HxV) | NTSC: 976x494, PAL: 976x582 |
| Resolution | 700 TVL |
| Min. Illumination | 0.02 Lux @ F1.2 |
| S/N Ratio | More than 50dB |
| Electronic Shutter | NTSC:1/60~1/100,000, PAL:1/50~1/100,000 |
| Gamma | 0.45 |
| Iris Control | Fixed |
| Lens Furnished | Cone Lens 3.7mm / F2.0 |
| PIR N.C/N.O Relay Output | 3A/120 VAC, 3A/24 VDC |
| PIR N.C/N.O Relay Alarm Time | 2 to 40 sec. (Adjustable) |
| PIR Detection Angle | Hor.=100° / Ver.=45° |
| PIR Detection Angle | Up to 15M |
| Sync. System | Internal |
| Video Output | 1 Vp-p / 75 Ohms |
| Power Supply | DC12V±10% |
| Power Consumption | 175mA |
| Operating Temp. | -10°C ~ 50°C |