Configure COM2 of VP-300 as RS-232 / RS-485 / RS-422

- Turn off power and open the case of VP-300.

- Find SW3 dip switch near the power switch.

- Adjust SW3 according to the mode as follows:

| RS-232 Mode |  | RS-485 Mode |  | RS-422 Mode |  |
| --- | --- | --- | --- | --- | --- |
| SW1 | OFF | SW1 | ON | SW1 | ON |
| SW2 | OFF | SW2 | OFF | SW2 | ON |
| SW3 | OFF | SW3 | ON | SW3 | ON |
| SW4 | OFF | SW4 | ON | SW4 | ON |
| SW5 | OFF | SW5 | ON | SW5 | ON |
| SW6 | OFF | SW6 | ON | SW6 | ON |
| SW7 | OFF | SW7 | ON | SW7 | ON |
| SW8 | OFF | SW8 | ON | SW8 | ON |

- Close the case and turn on the power.

- Test with device of the same communication protocol.