# WebMonitor Firmware 3.0 Specification

Highlights:

--- Half-duplex Audio Function

--- Audio inputs as triggering events

--- Toll Quality

Specifications:

- Cover all the specifications of Firmware 2.0, plus audio capability.

- The audio is operated at half-duplex mode. Client can only speak or just listen to WebMonitor but not at the same time.

- At the same time only one audio source can be selected to be active.

- Client has the option that mixes all the audio sources into one audio stream and makes it active. That is, there are 2 operating modes: single mode and mix mode.

- Client can adjust the output volume and microphone gain of WebMonitor.

- Audio stream and video function can not run simultaneously, when there is an audio stream opened, the opened video stream will be suspended until the close of audio stream. Client does not need to close the opened video stream but the video stream will be paused automatically.

- Audio inputs at WebMonitor site can be used as triggering events, client can adjust the triggering threshold values of these sources separately.

- Audio events and video motion-detect events can be watched at the same time. If there is an event (GPIs, Video Motion-detection, Audio input) triggered and a callback is set, the predefined video recording will be executed and all the other audio and video function is stopped.

- The quality of audio is toll quality.

- The delay of audio must not exceed 3 seconds.

- WebMonitor site require 33.6 K Modem or above. At minimum, Client site requires Pentium 200 with 32 MB RAM and running Windows 95. A sound card, a microphone and speaker or ear-phone are also required.