# VPON Modem Initialization String Comment

# Default initialization string: “AT&FE0V1Q0S0=0L0M0&D2&C1&S0X4”

&F: Load factory settings.

E0: Disable character echo in command state.

V1: Result code in textual form.

Q0: Modem returns a result code.

S0=0: Disable auto-answering.

L0: Speaker volume OFF.

M0: Speaker is always OFF.

&D2: DTR OFF causes modem to hang up and return to command state.

&C1: CD tracks presence of carrier.

&S0: DSR always ON

X4: Result code option.

Note:

- Make sure the default setting of the modem is using hardware flow control ( CTS/RTS flow control ). Otherwise you should explicitly add this AT command to initialization string.

- Default dial prefix: “ATDT”.