VPON Webpage Syntax v3.0

This document is applied to VPON Firmware v2.16 (VP200) and v3.0(VP300) or later version

Introduction:

To revise the web pages of VPON, you can first use a FTP application connect to VPON, download all the web pages under C:\WWW\USER to a directory in your host PC. Making a backup of the downloaded pages is a good idea. Then use a text editor to modify the pages. These pages are written in HTML, except for some tokens. These tokens is constructed with one pair of ‘<$’ and ‘$>, and a string enclosed by these two marks. For example: <$Date$> will be replaced with a string which is the system date of the system. These tokens will be read and parsed by firmware and will be replaced with another string, so that you can write your own webpages which will dynamically change the actual pages shown with parameters. Some tokens will contains one or more colon for one or more parameters.

These tokens are not complied to HTML, so it will not be so convenient to edit it with standard HTML editor. After revision, you can upload the pages to VPON using FTP and use browser to test the result.

It should be noted that if you find that some part of one page is absent abnormally, please first check if the ‘<$’ and ‘$>’ are in pairs.

Reference:

All the tokens related to audio function is applied to VP300 only.

Date

Current system date

Time

Current system time

GQStr

General query string, should be used when the link will direct to pages containing video

rsize?

Syntax:  <$rsize?string1:string2$>

If the value of rsize in query string is 1, then this expression will be replaced by string1. Otherwise, string2 will be used.

Example: <$rsize?Enlarge:Normal$> If rsize is 1, the result will be Enlarge, otherwise, Normal will be the result.

zoom?

Syntax:  <$zoom?string1:string2$>

If the value of zoom in query string is 1, then this expression will be replaced by string1. Otherwise, string2 will be used.

ServerName

Name of server

CameraName

Syntax: <$CameraName:k$>

Name of camera # k, k=0 means camera 1

IpAddr

Current System IP Address

GatewayIpAddr

Current Gateway IP Address

# SubnetMask

Current Subnet Mask

LiLinkCamCtrl

Syntax: <$LiLinkCamCtrl:k:string$>

A hyper link to camera control of camera # k, k starts from 0.

Example: <$LiLinkCamCtrl:2:Camera Control> == <li><a href=\"camcfg.htm?scrw=1024&Video_Source=2&zzw=359">Camera Control</a></li>

VidAutoWidth

VidAutoHeight

Automatically adjusted video window width/height, for user screen width 1024 or above case, if rsize is 1, then it is 640x480, otherwise it’s 320x240. If user screen width is lower than 1024, it’s 480x360 for rsize = 1 and 240x180 for rsize = 0.

VidAutoEnlargeWidth

VidAutoEnlargeHeight

Automatically adjusted enlarged video window width/height, for user screen width 1024 or above case, it is 640x480, otherwise it’s 480x360.

CameraNameSel

Camera name of the current camera selected in this page

PlgLoadPage

Link to download plugin page:

LiLinkCamera

Syntax: <$LiLinkCamera:k$>

A hyper link to camera # k, k starts from 0. If user has no aceess right for this camera, a empty string will be used.

Example: <$LiLinkCamera:2$> == <li><a href="source2.htm?scrw=1024">Camera 3</a></li>

CamCtrlToken

Current camera control access token, if it is not allocated, allocate a new one for it.

UserSel

Id of the current limited user

LimitUserNameSel

Name of the current limited user

LimitUserPassSel

Password of the current limited user

IfSel_User

Syntax: <$IfSel_User:k$>

If k is the id of the current user, “selected” will be used, otherwise a null string will be used.

IfSel_LimitUserCam

Syntax:<$IfSel_LimitUserCam:k$>

If current user has the access right of camera # k, “selected” will be used, otherwise a null string will be used.

IfSel_LimitUserAdo

Syntax:<$IfSel_LimitUserAdo:k$>

If current user has the access right of audio # k, “selected” will be used, otherwise a null string will be used.

RemOn_NotAvailVidSrc

Syntax:<$RemOn_NotAvailVidSrc:k$>

For the current user, first make a list of all the available camera that this user can access, then if current user has the access right of # k of the available camera, a null string will be used, otherwise “!” will be used. This is used to dynamically disable an HTML tag.

AvailVidSrc+1

Syntax: <$AvailVidSrc+1:k$>

For the current user, first make a list of all the available camera that this user can access, then if current user has the access right of # k of the available camera, this camera id increased with 1 will be used, otherwise 0 will be used. For example: If the cureent user can access camera 1, 3, 5, and 6. <$AvailVidSrc+1:2$> is 5 plus1, it’s 6.

AvailVidSrc

Syntax: <$AvailVidSrc:k$>

For the current user, first make a list of all the available camera that this user can access, then if current user has the access right of # k of the available camera, this camera id will be used, otherwise 0 will be used.

AvailCamName

Syntax: <$AvailCamName:k$>

For the current user, first make a list of all the available camera that this user can access, then if current user has the access right of # k of the available camera, this camera name will be used, otherwise a null string will be used.

RemOn_NotAvailAdoSrc

Syntax:<$RemOn_NotAvailAdoSrc:k$>

For the current user, first make a list of all the available audio that this user can access, then if current user has the access right of # k of the available audio, a null string will be used, otherwise “!” will be used. This is used to dynamically disable an HTML tag.

LiLinkLiveVu

Syntax: <$LiLinkLiveVu:k$>

A hyper link to live view camera # k, k starts from 0. If user has no aceess right for this camera, a empty string will be used.

Example: <$LiLinkLiveVu:2$> == <li><a href="livevu2.htm?scrw=1024">Camera 3</a></li>

VideoMimeType

The MIME string of the current video stream

VidSrcSel

Current video source in this page

CamCtrlDeviceID

Current camera control device ID number..

CamCtrlDevicePage

If the current user is administrator, “PTZxxxxA.HTM” will be used, otherwise “PTZxxxU.HTM” will be used. xxxx is the hexidicimal form of the current camera control device ID.

AudioMimeType

The MIME string of the current audio stream

AdoSrcSel

Current audio source in this page

IfSel_VidSrc

Syntax:<$IfSel_VidSrc:k$>

If current camera is camera # k, “selected” will be used, otherwise a null string will be used.:

IfSel_AdoSrc

Syntax:<$IfSel_AdoSrc:k$>

If current audio is audio # k, “selected” will be used, otherwise a null string will be used.:

IfSel_MicGain

Syntax:<$IfSel_MicGain:k$>

If microphone gain of the current audio source is k, “selected” will be used, otherwise a null string will be used.

IfSel_SpkVol

Syntax:<$IfSel_SpkVol:k$>

If speaker volume of the current audio source is k, “selected” will be used, otherwise a null string will be used.

EtherAddr

The current ethernet address, MAC address.

EtherIOAddr

The IO address of the current ethernet device.

EtherIRQ

The IRQ number of the current ethernet device.

EtherDevType

The devide type of the current ethernet device.

EtherDevID

The device id of the current ethernet device.

EtherDevFlag

The device flag of the current ethernet device.

FirmwareVersion

The current firmware version.

FirmwareDate

The built date of the current firmware.

SystemLog

Dump the system log

# CurrIpAddr

The same as IpAddr

IfChk_ISP_BAUD

Syntax:<$IfChk_ISP_BAUD:k$>

If the current baud rate used in ISP mode is k, “checked” will be used, otherwise a null string will be used.

IfChk_IspSetIP

Syntax:<$IfChk_IspSetIP$>

If the current setting is to use the assigned TCP/IP settings in ISP mode, “checked” will be used, otherwise a null string will be used.

IfSel_Com1Baud

IfSel_Com2Baud

Syntax:<$IfSel_Com1Baud:k$>, <$IfSel_Com2Baud:k$>

If the baud rate of com1 or com2 is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

Indexed baud rate:

0: 115200, 1: 57600, 2: 38400, 3:28800, 4:19200, 5:14400, 6:9600, 7:4800, 8:2400, 9:1200

IfChk_Com2HWFCtl

If the hardware flow control of com2 is set, “checked” will be used, otherwise a null string will be used.

IfSel_Encode_Algorithm

Syntax:<$IfSel_Encode_Algorithm:type$>

If video encoding algorithm is equal to type, “selected” will be used, otherwise a null string will be used.

The value of type can be JPEG, MJPEG, H263.

IfSel_Video_Resolution

Syntax:<$IfSel_Video_Resolution:k$>

k=1:640x480, k=2:320x240

If video dimension is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

# IfSel_CamCtrlType

Syntax:<$IfSel_CamCtrlType:k$>

If the current camera control type is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

IfChk_CamCtrlEn

Syntax: <$IfChk_CamCtrlEn:k$>

If the current camera control type is applied to camera # k, “checked” will be used, otherwise a null string will be used.

IfChk_MASTER

Syntax: <$IfChk_MASTER:k$>

If the current running mode is k, “checked” will be used, otherwise a null string will be used.

k- 1:Network Mode, 2:ISP Mode, 3:Viewer(RS232/DirectConnection Mode)

UserName

User’s name

UserPass

User’s password

AdminName

Administrator’s name.

AdminPass

Administrator’s password.

Com1InitStr

Initialization string for modem connected to COM1.

CamCtrlTimeout

Camera control access timeout.

ISP_PhoneNum

ISP telephone number

ISP_Login

User account to login ISP

ISP_Password

User password to login ISP

IfSel_Brightness

Syntax:<$IfSel_Brightness:k$>

If the current brightness is equal to the value of k, “selected” will be used, otherwise a null string will be used.

IfSel_Contrast

Syntax:<$IfSel_Contrast:k$>

If the current contrast is equal to the value of k, “selected” will be used, otherwise a null string will be used.

IfSel_Saturation

Syntax:<$IfSel_Saturation:k$>

If the current saturation is equal to the value of k, “selected” will be used, otherwise a null string will be used.

IfSel_Hue

Syntax:<$IfSel_Hue:k$>

If the current hue is equal to the value of k, “selected” will be used, otherwise a null string will be used.

IfSel_Quality

Syntax:<$IfSel_Quality:k$>

If the current quality is equal to the value of k, “selected” will be used, otherwise a null string will be used.

IfSel_InpGain

Syntax: <$IfSel_InpGain:k$>

If the current input gain is equal to the value of k, “selected” will be used, otherwise a null string will be used.

IfSel_TstampDate

Syntax: <$IfSel_TstampDate:k$>

k- 0:DD/MM/YYYY. 1:MM/DD/YYYY

If the current time stamp format is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

IfSel_GPI

Syntax: <$IfSel_GPI:k1:k2$>

k2- 0:NO,1:NC

If the current GPI # k ( starts from 0) is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

IfSel_GPO

Syntax: <$IfSel_GPO:k1:k2$>

k2- 0:OFF,1:ON

If the current GPO # k ( starts from 0) is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

IfSel_CamCtrlSpeed

Syntax: <$IfSel_CamCtrlSpeed:k$>

k- 0:slow, 1:normal, 2:fast

If the current camera control speed is equal to the value of index k, “selected” will be used, otherwise a null string will be used.

IfChk_GCEN

If gain control of the selected camera in this page is enabled, “checked” will be used, otherwise a null string will be used.

IfChk_AGCEN

If automaically gain control of the selected camera in this page is enabled, “checked” will be used, otherwise a null string will be used.

IfChk_HMirr

If horizontal mirror of the selected camera in this page is enabled, “checked” will be used, otherwise a null string will be used.

IfChk_VMirr

If vertical mirror of the selected camera in this page is enabled, “checked” will be used, otherwise a null string will be used.

IfChk_OTextEn

Syntax: <$IfChk_OTextEn:k$>

If the overlay text is enabled for camera # k ( starts from 0), “checked” will be used, otherwise a null string will be used.

IfChk_TstampPos

Syntax: <$IfChk_TstampPos:k1:k2$>

k2- 0:disable, 1:upper left, 2:upper center, 3:upper right, 4:lower left, 5:lower center, 6:lower right

If the time stamp position for camera # k1 is at position of index k2, “checked” will be used, otherwise a null string will be used.

IfChk_PluginUrlEn

If plugin url is enabled, “checked” will be used.

OTextName

Syntax: <$OTextName:k$>

The overlay text name of camera # k

OTextX

Syntax: <$OTextX:k$>

The overlay text x coordination of camera # k

OTextY

Syntax: <$OTextY:k$>

The overlay text y coordination of camera # k

GPI_State

Syntax: <$GPI_State:k$>

The GPI state is 1, “ON” will be used, otherwise a null string will be used.

PluginUrl

Plugin download url

RegIpAddr

Regidstry server IP address

RegPath

Registry server path

RegDomain

Registry host domain

MaxVideoConnectionSel

The max connections allowed for this camera

MaxVideoBandwidthSel

The max bandwidth allowed for this camera

MaxConnectionBandwidthSel

The max bandwidth allowed for each connection of this camera

MicTimeout

Microphone access timeout value

SpkTimeout

Speaker access timeout value

PLGIE_EXE

If current alogrithm is H.263, ir will be “NW3IE.EXE”, otherwise, it will be “NWJIE.EXE”

“NW3IE.EXE” is a self extracting setup file for h.263 decoder plugin for IE

“NWJIE.EXE” is a self extracting setup file for jpeg decoder plugin for IE

You can put a newer version onto VPON through FTP server to replace the older version.

PLGNN_EXE

If current alogrithm is H.263, ir will be “NW3NN.EXE”, otherwise, it will be “NWJNN.EXE”

“NW3NN.EXE” is a self extracting setup file for h.263 decoder plugin for IE

“NWJNN.EXE” is a self extracting setup file for jpeg decoder plugin for IE

You can put a newer version onto VPON through FTP server to replace the older version.

PLGIE_EXE_SIZE

The file size of PLGIE_EXE

PLGNN_EXE_SIZE

The file size of PLGNN_EXE

BootDevice

The media that the system booted from

### MDAreaX

The x coordinate of the top-left corner of the motion detection area

### MDAreaY

The y coordinate of the top-left corner of the motion detection area

### MDAreaW

The width of the motion detection area

### MDAreaH

The height of the motion detection area

### IfSel_mden

Syntax: <$IfSel_mden:k$>

k- 0:disable,1:enable

If the motion detection function of the current selected camera is enabled/disabled as k indicated , “selected” will be used, otherwise a null string will be used.

IfSel_mdsv

Syntax: <$IfSel_mdsv:k$>

If the motion detection sensitivity of the current selected camera is the same as k, “selected” will be used, otherwise a null string will be used.

IfSel_mdben

Syntax: <$IfSel_mdben:k$>

k- 0:disable,1:enable

If the display of the motion detection area of the current selected camera is enabled/disabled as k indicated , “selected” will be used, otherwise a null string will be used.

FW3.02 or later

IfChk_Com2Data

Syntax: <$IfChk_Com2Data:k$>

k- 8: 8 data bits, 7: 7 data bits, 6:6 data bits

If the length of data bits of COM2 is the same as k, “checked” will be used, otherwise a null string will be used.

IfChk_Com2Stop

Syntax: <$IfChk_Com2Stop:k$>

k- 0: 1 stop bit, 1: 2 stop bits

If the length of stop bits of COM2 is the same as the value indicated by k, “checked” will be used, otherwise a null string will be used.

IfChk_Com2Pari

Syntax: <$IfChk_Com2Pari:k$>

k- 0: no parity, 1: odd parity, 2: even parity

If the parity setting is the same as the value indicated by k, “checked” will be used, otherwise a null string will be used.

FW3.03 or later

RegSvrListCount

The visible records count on registry server.

RegSvrListItem

Syntax: <$RegSvrListItem:k1:k2$>

k1- start index of record to be listed, k2:count of records

Start output a list of records of count k2 and fill these records into text form fields, start from index k1 of records in list

RegSvrListItemSel

Syntax: <$RegSvrListItemSel$>

Start output a list of records of count k2 and fill these records into text form fields, start from index k1 of records in list, where k1 and k2 are specified in query string as: “xxxx.htm?start=k1&count=k2”

RegSvrListItemStartSel

Syntax: <$RegSvrListItemStartSel$>

Get the number of k1, where k1 is specified in query string as: “xxxx.htm?start=k1”

RegSvrListItemCountSel

Syntax: <$RegSvrListItemCountSel$>

Get the number of k2, where k2 is specified in query string as: “xxxx.htm?count=k2”

RegSvrQueryResult

Syntax: <$RegSvrQueryResult$>

Get the IP address that queried. The query string may contains “ETHADDR=MacID_To_Be_Queried” or “NAME=Name_To_Be_Queried”

RegSvrQueryMacID

Syntax: <$RegSvrQueryMacID$>

Get the Mac ID contained in the query string. Ex:“ETHADDR=MacID_To_Be_Queried”

RegSvrQueryName

Syntax: <$RegSvrQueryMacID$>

Get the Name contained in the query string. Ex:“NAME=Name_To_Be_Queried”

IfSel_RegSvrEn

If registry server function is enabled, “selected” will be used, otherwise a null string is used.

CameraPresent

Syntax: <$CameraPresent:k$>

If camera # k is detected, 1 will be used, 0 otherwise.

FW3.041 or later

RemOn_Admin

Syntax:<$RemOn_Admin:k$>

If the current user is administrator: k=0, “!” will be used. k=1, null string will be used.

If the current user is not administrator, k=0, null string will be used. k=1, “!” will be used.

This can be used to comment off an HTML tag or apply with <comment></comment> to comment off a segment of HTML code.

RemOn_User

Syntax:<$RemOn_Admin:k$>

If the current a user is a valid user ( administrator is also a valid user ): k=0, “!” will be used. k=1, null string will be used.

If the current not a valid user, k=0, null string will be used. k=1, “!” will be used.

This can be used to comment off an HTML tag or apply with <comment></comment> to comment off a segment of HTML code.

UserSecurCode

Syntax:<$UserSecurCode$>

The result will be “0” if this is not a valid user, “1” if is a user, “3” if is a limted user, “2” if is a administrator.

## LANGUAGE.TXT

There are some messages, names, strings, etc. that need to be customized for different langauge but can not be defined in web pages. We pack these strings into LANGUAGE.TXT. When VPON starts, it will read this file located in C:\ of VPON and read the strings and use then in runtime. To customize these resources, download this file and modify it using a text editor, DO NOT REMOVE ANY BLANK LINE, it is not meaningless. If the string contains ‘%’, like ‘%d’ ‘%ld’ ‘%X’ do not change them, otherwise the system may malfunction.

| 0. STR1_SAVE_REBOOT_WAIT | ---System now is saving config data and will reboot, please wait at least 1 minute. |
| --- | --- |
| 1. STR1_BACK_TO_HOME_PAGE | Back To Home Page --- Currently please left this line to be blank |
| 2. STR1_CAMCTRL_OCCUPIED | Camera control device currently is occupied (%d seconds left ). --- DO NOT CHANGE OR REMOVE ‘%d’. |
| 3. STR1_CAMERA | Camera --- This is used when there is no camera name assigned, VPON will append the camera id after this string as the default camera name. |
| 4. STR1_CCD_NONE | No Camera Control Device |
| 5. STR1_VP202 | VP-202 Camera Control |
| 6. STR1_VP203 | VP-203 Camera Control |
| 7. STR1_CONON_VCC3 | Canon VC-C3 Camera Control |
| 8. STR1_LILIN_PIH717X | PIH-717X Camera Control |
| 9. STR1_ACQUTEK_DOM1200A | Acqutek DOM-1200A Camera Control |
| 10. STR1_PELCO_D | Pelco (D) Camera Control |