# WebMonitor Firmware 1.4 Specification

Highlights:

--- Neutral Look And Feel

--- Keep Last Images of the Failed Cameras

--- Configurable Date/Time of WebMonitor

--- Multiple H.263 Video Streams

--- Configurable Dialing Behavior Of Callback

--- Naming Cameras And WebMonitor Under Dial-In Mode

Specifications:

- Cover all the specification of Firmware 1.3

- There is no sign or logo of any specific company, the looking of this version of firmware is neutral and general.

- User can see the last image of a camera if the camera is failed during a connection.

- There can be multiple H.263 video streams.

- Client can set Date/Time of WebMonitor and modem initialization string. Under Dial-In mode, client can also set the names of WebMonitor and all the cameras.

- When events triggered, server should record the videos and wait for W seconds (which can be configured by client), and then dial the pre-defined phone number. If the call is accepted, server will send a callback indication. Client then is responsible for downloading the recorded file. Client can also remove the file or break the download at any time. The recorded file is a single continuous file which can only be downloaded from the beginning. After successful download, the file is remained in WebMonitor until client removes it.

- If the call is not accepted, server should retry dialing every T minutes N times, if the call is still not accepted, another phone number should be called, there can be 3 set of pre-defined callback phone number. If all these trial failed, stop trying callback. T and N can be configured by client.