# WebMonitor Viewer 1.4 Specification

Highlights:

--- Keep Last Images of the Failed Cameras

--- Configurable Date/Time of WebMonitor

--- Multiple H.263 Video Streams

--- Configurable Dialing Behavior Of Callback

--- Naming Cameras And WebMonitor

Specifications:

- Cover all the specification of Viewer 1.0

- There is no sign or logo of any specific company, the looking of this version of viewer application is neutral and general.

- User can see the last image of a camera if the camera is failed during a connection.

- There can be multiple H.263 video streams.

- User can set Date/Time of WebMonitor and modem initialization string. Also user can set the names of WebMonitor and all the cameras. These names can only be configured by administrator and are recorded in WebMonitor.

- The relationship between GPI and GPO now is no longer exist. GPI can only trigger the callback but cannot control GPO. User can turn ON or OFF the GPO remotely.

- When events triggered, server should record the videos and wait for W seconds (which can be configured by client), and then dial the pre-defined phone number. If the call is accepted, server will send a callback indication. Client then is responsible for downloading the recorded file. User can also remove the file or break the download at any time. The recorded file is a single continuous file which can only be downloaded from the beginning. After successful download, the file is remained in WebMonitor until user removes it.

- If the call is not accepted, server should retry dialing every T minutes N times, if the call is still not accepted, another phone number should be called, there can be 3 set of pre-defined callback phone number. If all these trial failed, stop trying callback. T and N can be configured by user.